Computer Architecture - CS2323. Autumn 2025

Quiz-4 B

Total marks: 15                       Date: 16-10-2025                    Allowed Time: 15 min.

-------------------------------------------------------------------------------------------------------------------------------------

Student Name: ___________________________________________

Roll number: __________________________

- Which of the following DOES NOT affect the cost of a die [2 marks]

  - Die area

  - Processor being RISC or CISC

  - Average CPI of the processor

  - Yield

- Which of the following sequences can cause pipeline hazards? [2 marks]

  - add x4, x3, x5
ld x3, 0(x7)

  - ld x3, 0(x4) 
add x4, x3, x5

  - add x3, x4, x5 
add x5, x4, x8

  - add x4, x3, x5
beq x5, x4, LABEL

- A direct mapped cache has capacity of 8 blocks, block size=16 Bytes. Order of access:

0x2453, 0x2326, 0x0106, 0x245C, 0x2458, 0x2431, 0x2328, 0x2450, 0x0000, 0x2323, 0x1212, 0x1011. Identify which are hits and misses. The cache is empty at start. [6 marks]

- Processor choices: There are two processors PA and PB operating at 1 GHz (109 Hz) and 2 GHz, respectively. These processors support different types of instructions with their individual cycles per instruction (CPI) being shown in the table.

  - Which processor (PA or PB) is faster and by what factor? [3 marks]

  - What is the execution time for the faster processor to execute 1010 instructions? [2 marks]

| Instruction type | CPI for PA | CPI for PB | Fraction of instructions |
| --- | --- | --- | --- |
| Basic ALU | 1 | 1.5 | 0.60 |
| Mult | 3 | 5 | 0.20 |
| Load-store | 2 | 4 | 0.10 |
| Others | 3 | 4 | 0.10 |