COMPUTER ARCHITECTURE-SCRIBE

IO Systems

DATE:4-November

Kodadala.Jagadeesh(AI23BTECH11012)

Sura.Venkata Sailaja(AI23BTECH11029)

J.Devisree Chandana(AI23BTECH11008)

Jatavath.Ajay(AI23BTECH11011)

Recap:

Until now in this course we have discussed processor design, ISA, Pipelining, Memory Hierarchy, and Caches so far. Now  we are going to learn about input and output devices.

Think about:

- Can we have a system without input and output devices?

No, in general. Such a system would have no significance. Think of a scenario where you have a highly efficient system without being able to give input or get output. We won’t be able to utilise the system. Therefore input and output devices are equally important as a good processor or a cache for a system to be useful.

- Would only an input or output device suffice?

Wouldn’t be that useful for most useful systems both input and output devices are required.

I/O in general are slower as compared to processor or memory. They mostly work at human speed, so they need to be handled in a different manner.

Example:

Input only: Mouse, KeyBoard, Sensor etc..,

Output only: Monitor, Speaker etc..,

Both input and output: Network Port, Discs, TouchPad etc…,

- Where would these fit in a pipelined processor?

In the case of the LED experiment we interacted with the RED-V board using memory adresses.

This is one way of interaction of I/O devices i.e., by reserving a set of memory addresses. In the pipelined model these fit in parallel with the memory.

In general I/O devices has three types of registers:

- Control Registers: It helps us configure the device in a particular mode. For example:In a monitor, it specifies at what frequency it should refresh or resolution of a camera. We can read or write to control registers to know the device setting or change the device settings.

- Status Registers: These registers convey the status of the device.	It specifies whether the system is ready to send to the data the processor .Its Functionality is context specific(displaying an error message / overflow) .It can be read or written by the processor. Depending on the device, it  can have separate control and status registers or both merged.

- Data registers: It is the buffer/storage within the device that stores the data given by processor or user. It can have multiple registers.

MEMORY MAPPED I/O:

Reserving a set of memory locations for I/O devices and accessing them using load/store instructions.

The issue that comes up:

Typically, caches would mark an address as valid if it is present in the cache assuming that no one else would modify the content of this address location. Now that the IO device can change the value of its registers without any involvement from the processor, the cache might still return stale data. Or, in a write-back cache scheme, the data might not get written to the actual device register as it might be available in the cache and this might cause device malfunction. Hence, caches must be bypassed for the IO device accesses. Such a bypassing is achieved by indicating some portion of the memory addresses as non-cacheable (supported internally during processor design).

I/O  MAPPED I/O:

Older systems follow another different method for handling the I/O devices due to lack of large memory (present systems using 64-bit registers) ; it is an I/O mapped I/O method.

This method requires extra instruction for input and output

IN :                Registers   ←    Address

OUT:                Registers   →    Address

Here we don’t have issues with caching.

- How does the processor know that it got input from the user??

Polling: Periodically check after a certain number of instructions for each I/O and update the status register for any change.

- Analogous to a professor asking each student whether he has any doubts or not.

Overheads:

- May lead to control of Hazards.

- Wastage of time. Effective amount of work done will be decreased if we period is decreased  due to unnecessary checking incase of increased period response time will increase which is not desiarable.

- In case of multiple I/O devices overheads increase significantly.

Interrupt: Whenever an I/O device has a status update it gives a signal to the processor. The processor chooses to respond to the signal or not . The CPU  can choose to disable the interrupt when running a program of more priority. An interrupt is more efficient for slow devices.

I/O devices have a control bit that determines whether/ not to enable/disable interrupts.

Overheads:

- An interrupt can be raised at any point of time i.e., it is unplanned.

Therefore caller-callee convention cannot be done. So, everytime an interrupt is raised, the processor has to save all the registers.

- Leads to more control hazards, as compared to polling.

In RISC-V, everytime an interrupt is raised, the PC is saved in a special register called SEPC [Supervisor Exception Program Counter] and the cause of the interrupt is stored in SCAUSE [Supervisor Exception Cause register]. SCAUSE is a 64-bit register in which most of the bits are unused, it contains the exception code field i.e., it stores the reason for different kinds of interrupts. For example: 2 for undefined opcode, 12 for hardware malfunction, etc.

Handler contains codes that get executed for each kind of interrupt. Whenever an interrupt is raised, the instruction jumps to the location of the handler (a pre-defined address) to process the exception.

Restartable exceptions: Pipeline can flush the instructions, then handler executes, returns back to the instructions, that is refreshed and executed from scratch.

Priority is implemented if multiple devices raise an interrupt at the same time. A simple approach is to flush subsequent instruction so that precise exceptions can be maintained.

In case of complex pipelines, multiple instructions are issued per cycle due to which, we have out-of-order completion, therefore maintaining precise exceptions is difficult.

Trap: It is also a kind of interrupt but of higher priority. It is of the highest priority. It is difficult to disable.

Changes required in the processor design to support interrupts:

- There needs to be some connection between the IO device and the processor which can inform the processor about the device being ready. We call this line as an INTERRUPT signal or the INTERRUPT input.

- Internally, when deciding the next PC through a MUX, we add one more condition that if it is an interrupt signal being high, the PC should jump to a pre-defined address (X).

- The current PC should be saved in the SEPC register automatically and the register should be readable by the instructions.

- A pipelined processor should flush the pipeline and start executing from the new location (X) on receiving an interrupt.

Think about:

- Imagine  a scenario where you connect a pen-drive to your system and want to copy data from the drive to the system.What would the processor do in this case???

We can't waste valuable processor time by using multiple store and load statements to copy such huge chunks of data.Here comes the role of DMA [Direct Memory Access]. This used to just copy and store the data from a device to memory or from memory to IO device.

CPU merely configures the DMA after which it continues with its execution.

When DMA is required the processor is going to give the start and end address of the memory to be copied and D-start and D-end address which corresponds to the destination of the memory to be stored. This is used for larger chunks of data; it is not feasible for smaller chunks of data.

Other references:

- Hennesey & Patterson book, Sections 1.4, 4.9, 5.2, 5.5, 5.11, and 6.9.

- Basic computer architecture, Smruti Ranjan Sarangi, Chapter-12. https://www.dropbox.com/s/q0h0ljmqxporbo5/Chapter-12.pdf?dl=0