ISA (Instruction Set Architecture)

- ISA forms the interface (set of rules that help two or more things work together – here the common language that both the hardware and software understand is the ISA) between the software and hardware of a computer

- It defines the computing facilities offered by the hardware

- Two predominant types: RISC and CISC

| RISC | CISC |
| --- | --- |
| Reduced Instruction Set Architecture | Complex Instruction Set Architecture |
| Operates on a load-store architectural model | Supports memory operands |
| Uniform-length instructions | Variable-length instructions |
| Fewer number of instructions | Greater number of instructions |
| eg. add r1, r2, r3 // r1 assigned value of r2 + r3 | eg. add r1, r2, *r3 // *r3 comes from memory, not registers |
| Example: RISC-V, ARM, MIPS | Example: Intel, AMD, IBM370 |
| Primarily used in embedded (special-purpose computer that is built into a larger system or product to perform dedicated functions)/handheld devices (Phones use RISC as it is simpler, reduces power consumption) | Primarily used in desktop devices/servers, laptops. |

- Early computers had very simple ISAs, as do many modern RISC devices.

- It’s theoretically possible to have an ISA with only 5-6 instructions (Ex: An ISA with only the following instructions: LOAD (move data from memory → register); STORE (move data from register → memory); ADD (add two registers); SUB (subtract two registers); JUMP (go to another instruction); BRANCH-IF-ZERO (conditional jump)), but in practice smaller ISAs use around 20 or more types of instructions.

- Sometimes, backwards compatibility may compel the inclusion of instructions that may not make logical sense to have; these are retained to prevent newer systems from failing to run older/legacy programs.

- RISC != RISC-V

- RISC     : A generic term for RISC type of ISA

- RISC-V : One Type of RISC architecture

RISC-V

- RISC-V was developed at the University of California, Berkeley, as an open-source ISA.

- Open-source ISA implies that anyone can build systems using this ISA, unlike proprietary (owned and controlled by a particular company or individual) ones like Intel or ARM.

- RISC-V can be used in various embedded applications.

- Indian RISC-V initiatives include:

- Shakti series of processors (developed in IITM)

- Digital India RISC-V (DIR-V) initiative

- Vega processor from CDAC (government-funded)

- RISC-V has 2 types of instructions: Unprivileged and Privileged.

- Unprivileged instructions are used for general applications, and are more commonly used. Ex: add, sub, etc.

- Privileged instructions are required for specific operations needed by the Operating System (OS). These are restricted instructions that can only be executed in kernel/supervisor mode. They directly affect hardware, system control, or memory protection. If a user program tries to run them → CPU raises an exception (trap). Ex: hlt – halt CPU operations.

- RISC-V supports 3 different widths registers: 32-bit (RV32), 64-bit (RV64), and 128-bit (RV128).

- Most instructions remain the same independent of whether they are RV32, RV64, or RV128. A few RISC-V instructions may vary depending on the type of registers used.

Standard terms for different Data Sizes

- Bit(b): Single binary digit (0 or 1)

- Byte(B): 8 bits

- Word(W): 32 bits (4B)

- Halfword(H): 16 bits (2B or 1/2W)

- Doubleword(D): 64 bits (8B or 2W)

- Quadruple word(Q): 128 bits (16B or 4W)

Basic operands

- Add and subtract operations in RISC-V have 3 operands : Two for source and one for destination

- For example :

add a, b, c  //  a <= b+c

sub a, b, c  //  a <= b–c

- This follows with the design principle “Simplicity favors regularity”, i.e. with simple design we can get higher performance at lower cost.

Registers

- RV64 has 32 registers (each of 64 bit) named x0, x1 .. till x31.

- As mentioned above, RISC has load store architecture. It can do arithmetic operations only on registers.

- Naturally, registers are used for frequently accessed data.

- The limited number of registers follows the design principle “Smaller is faster”, i.e. larger sized memory (like DRAM) will be much slower.

Operations with constants

- To add constants to the register, we have the addi command which works as follows:

addi x22, x21, 1 //   a <= b + 1

(Note : you don’t need a ‘subi’ command, as negative constants are allowed in addi)

- Small constants are stored in the register, which follows from the design principle “Make the common case fast”

Handling larger data structures

- Arrays, dynamic data structures, etc are stored in the main memory. Whenever the need arises to operate on them, we can load the data into the register and perform the required operations on them.

- Memory is byte addressable. (Implies every single byte in memory has its own address)

- Larger data elements like word(32 bytes) or doubleword(64 bytes) occupy multiple addresses.

- Thus, to go from A[0] to A[4], where A[0]’s address is stored in p register and each element is a word(4 bytes), we need to do p+4*4 = p + 16 and not p+4 (which would go to p + 4/4 = p+ 1 => A[1] and not A[4]).

- We can access every element of an array as an offset of base.

Endian-ness

- There are two ways in which we can classify computers :

- Big endian

In this arrangement, the most significant address is stored in the least byte address. For example, a short integer with 2 bytes with the binary value

‘01001010 11100011’ would be stored as:-

Address |  Value

0	      01001010

1	      11100011

- Little endian

In this arrangement, the least significant byte is stored at the least byte address.

For example, the same number would be stored as:-

Address |  Value

0	      11100011

1	      01001010

- RISC-V and many other systems commonly follow Little endian. However, there are processors with Big Endian as well.

- Endianness does not affect functionality as long as both reads and writes follow the same interpretation (byte-ordering scheme). When reading a smaller number of bytes from a data element, we should be aware of endianness to read the correct byte.