# LECTURE-4

CS23BTECH11049 - Ramanolla Shravani

CS23BTECH11018 - Dondeti Samhitha

## Conditional Instructions:

The default nature of a CPU is to execute assembly code line by line, in the sequential order of addresses. But in a high-level programming language, we use loops, conditional  statements (if-else), etc. to make our code concise as well as to make the code scalable to perform larger operations. In such situations, the code does not always run sequentially and might skip certain statements or go back to a previous statement and execute them continuously. So, we need a method to transfer execution from one line to any other line (not necessarily the next line of instruction). In RISC-V, we have conditional instructions that make it possible. These instructions are similar to a goto construct in C language.

Let’s discuss two simple conditional instructions (or branch instructions):

- beq rs1, rs2, L1

beq stands for branch if equals. This instruction transfers the control to a line that has label L1 if register rs1 equals rs2. Otherwise, the control proceeds sequentially to the next line.

- bne rs1, rs2, L1

bne stands for branch if not equal. This instruction transfers the control to a line that has label L1 if rs1 is not equal to rs2. Otherwise, the control proceeds sequentially to the next line.

Both these instructions are called conditional branches. These instructions test values and transfer the control to a new line according to the test outcome. The label L1 can be above or below the Instruction step, but it shouldn't be relatively far from the current line. These labels are actually converted as address offsets by the assembler.

Example:

C code :

if ( i == j )

f = g + h ;

else

f = g - h;

Here i, j, f, g, h are corresponding to x19, x20, x21, x22, x23 registers.

RISC-V code :

bne x19, x20, ELSE    // if i is not equal to j goto ELSE

add x21, x22, x23       // f = g + h (this instruction is skipped if i not equal to j )

beq x0, x0, Exit          // Unconditional branching to label EXIT

ELSE : sub x21, x22, x23   // f = g - h (this instruction is skipped if i = j)

EXIT :  …….

An unconditional branch is realized using beq with the same registers like beq x0, x0, L1

We can also execute loops using these instructions. There is no separate instruction to handle loops. Just like if else, we use branch instructions to skip some code; we go back to an earlier line in the case of loops.

Example :

C Code :

while( i != k )

i += 1;

i, k corresponds to x22, x23 registers.

RISC-V Code :

LOOP : beq x22, x23, EXIT   // if i equals k then goto EXIT label

addi x22, x22, 1        // i = i + 1 (skips if i equals k)

beq x0, x0, LOOP     // Unconditional branching to LOOP label

EXIT :  ……

Example :

C Code :

while(A[ i ] ==  k)

i += 1

i, k corresponds to the registers x22, x24. The base address of A is stored in the register x25.

RISC-V Code :

LOOP : slli x10, x22, 3             // x10 = x22*8 = 8*i

add x25, x25, x10       // x25 = x25 + 8*i –address of ith element of the array A

ld x9, 0(x25)                // load A[ i ] to x9

bne x9, x24, EXIT       // if A[ i ] not equal to k then goto EXIT

addi x22, x22, 1          // i += 1 (skips this instruction if above one is true)

beq x0, x0, LOOP       // Unconditional branching to LOOP label

EXIT : ……..

Unlike previous instructions which check for equality conditions, there are additional branch instructions to check for inequalities, as below:

- blt rs1, rs2, L1

blt stands for branch if less than. This instruction transfers the control to a line that has label L1 if rs1 is less than rs2. Otherwise, the control goes to the next line.

- bge rs1, rs2, L1

bge stands for branch if greater than or equal. This instruction transfers the control to a line that has label  L1 if rs1 greater than or equal to rs2. Otherwise, the control goes to the next line.

There are no instructions for greater than (bgt) and less than or equal to (ble). Because these can be achieved by exchanging rs1 and rs2 in less than and greater than or equal to instructions respectively.

bgt rs1, rs2, L1 can be achieved using blt rs2, rs1, L1

While comparing registers, to get correct results we should be aware of the signed or unsigned nature of the values. The instructions blt and bge consider values as signed. To support unsigned comparison, we also have instructions bltu (branch if less than, unsigned), bgeu (branch if greater than or equal, unsigned) instructions. These are similar to blt, bge instructions respectively but assumes the values in rs1, rs2 as unsigned values. Whereas blt, bge assumes the values in rs1,rs2 are signed values.

## Jump Instructions:

In high level programming languages, we use function calls, throw (Used when an error or exceptional situation happens. It throws an exception object (like an error message or error code)) catch blocks etc ..  , which might transfer the control to the line which may be relatively far and sometimes transfers the control to a code in a different file. Also, we need to come back to the original point of a function call after the function completes its execution. If we do an unconditional branch to a function, we have no mechanism to come back to the original point.

To achieve this, we have two more instructions called jump operations jal, jalr.

- jal rd, L1

jal stands for jump and link. This instruction stores PC+4 value in register rd. And transfers the control to a line that has label  L1.

Here PC stands for program Counter which stores the address of the current executing instruction. Generally, this instruction is used for function calls. PC+4 is the address of the next instruction (In RISC-V each instruction is 4 bytes) and hence is nothing but the return address after the function call. Saving the return address in register rd can help us come back to the original point by using that register.

- jalr rd, n(rs1)

jalr stands for jump and link register. This instruction stores PC+4 value in the register rd and transfers the control to the instruction at address rs1+n. Generally, this instruction is used to return from a function. In that case n will be kept 0 and rs1 is the return address that is saved during function call by using instruction jal. In many cases, there will be no use of the address that is stored in rd in this case and thus, we can use rd as so that we do not overwrite any register. However, in special cases like function pointers, etc., we might use another register to store the return address in rd. Most return from functions use jalr x0, 0(rs1).

Example:

C Code:

| //function call f1() | //function definition void f1( ) { ……… ……… } |
| --- | --- |

Assembly Code:

| #function call jal x7 f1 | #function Definition f1: …………… …………… jalr x0 0(x7) |
| --- | --- |

Byte/Halfword/Word Operations:

- Load byte/halfword/word :

- lb  rd , offset(rs1)

- lh  rd , offset(rs1)

- lw  rd , offset(rs1)

These instructions take particular bytes of memory (1 byte for lb, 2 bytes for lh and 4 bytes for lw) starting from address rs1+offset and place them at LSB of register rd. And remaining bytes in register rd are sign extended.

- Load byte/halfword/word unsigned:

- lbu  rd , offset(rs1)

- lhu  rd , offset(rs1)

- lwu  rd , offset(rs1)

These instructions take particular bytes (1 byte for lbu, 2 bytes for lhu and 4 bytes for lwu) of memory starting from address rs1+offset and place them at LSB of register rd. And remaining bytes in register rd are zero extended .

- Store byte/halfword/word :

- sb  rd , offset(rs1)

- sh  rd , offset(rs1)

- sw  rd , offset(rs1)

These instructions take particular bytes (1 byte for lb , 2 bytes for lh and 4 bytes for lw) from register rd starting from LSB and store those bytes in the memory starting from the address rs1+offset .

Special Register :

x0 is a special register that always returns its value as zero. x0 is also called the zero register. The writes to x0 register are ignored.

Uses :

- It can be used to initialize registers.

- Can be used for comparing with zero.

- It can be used to ignore the writes in instructions.

Pseudo Instructions :

These instructions enable simpler assembly code but are not implemented in the machine code. These instructions are translated to other instructions.

Examples :

| Pseudo Instruction | Meaning | Actual Instruction |
| --- | --- | --- |
| mv rd, rs | Move/copy value from register rs to rd | addi rd, rs, 0 |
| neg rd, rs | Negate the register rs and store in rd | sub rd, x0, rs |
| j label | Jump to a label without storing the PC (goto in C) | jal x0, label |
| li rd, 1000 | load immediate a value 1000 into rd | addi rd, x0, 1000 |

These are shortcuts to copy, negate, jump to a new line and initialize a register.

li can also be used for large constant numbers whereas addi can't.

RISC-V Extensions:

RISC-V is defined as a base class containing basic instructions that are needed to perform any calculation. But there are extensions which enable a better, efficient implementation of commonly used operations.

- I   : Base Integer class

- M : Includes Integer multiply , divide , remainder operators

- A : Includes Atomic memory operations for multiple CPU systems. (Instructions for atomic memory operations (read-modify-write memory in one step)). Ex: AMOADD.W.

- F : Includes Single precision floating point(32 bit) operations. Ex: FADD.S

- D : Includes Double precision floating point (64 bit) operations. Ex: FADD.D

- C : Includes compressed instructions (i.e. 16 bit encoding for frequently use instructions). Eg: Normal 32-bit ADD x8, x9, x10 while Compressed 16-bit C.ADD x8, x9, x10.

- G : General class consists of I,M,A,F,D classes. By convention, RISC-V defines G as a shorthand for the most commonly used extensions together.