Computer Architecture - CS2323. Autumn 2025

Lab-3 (alternate proposal) (Creating FPU HW)

-----------------------------------------------------------------------------------------------------------------

Design a hardware to create a Floating Point Unit, synthesize, and run it on a board.

Supported specifications:

- FP32, FP64 Add, Sub, Mul

- (Optional) Div, Sqrt

- 32 x 64 bit register file

- Will support exceptions in the form of a register output. (INV, DZ, OVF, UNF, INX)

- Will support 5 modes of rounding (RNE, RTZ, RUP, RDN, RNA)

- Custom Instruction formatting for input (reduced instruction length)

- (Optional) Will be pipelined (Pipeline length may be different operation to operation)

- Should interface with some peripheral device (onboard), so as to transmit and receive data. (Example: Python interface for a Pynq Board, custom UART controller for Arty-A7)

Will support the following instructions

- fadd

- fsub

- fmul

- ld

- sd

- feq, flt, fle

Can use verilog, or C HLS.

(Future improvements : Add coprocessor interface to be able to add to a RISC-V processor later)

Optional: Show Taylor series computation using your own FPU design

MORE DETAILS:

- FPU should have a similar interface as an ALU.

- The designed FPU should have 2 input ports to take the two input numbers, and generate one output number as result.

- We can have an additional port of relevant bits which decides the operation to be done.

- The register file can be outside the FPU and not a part of it, just like we have the standard ALU.

- The verification on an FPGA board must also be planned - how would you provide the inputs and outputs for testing the functioning?

Submission instructions:

- Submit a link to a github repo containing the following

- Verilog source / C source

- Demo Video of the hardware working

- Python / Verilog Test vector gen and verification code

- Design documentation? (as a pdf or readme)

- The assignment should be done individually

- Copying from others or any other source is strictly prohibited and subject to strict penalty. Generating code from ChatGPT is also NOT allowed. Please read https://cse.iith.ac.in/academics/plagiarism-policy.html for the department's policy against any form of copying.

- Assignments will be tested for similarity among each other and any violation will be handled appropriately.